THE GREAT CHEESE HUNT

A NEW MICROMOUSE EVENT

TECHNICAL BULLETIN #7

PUBLICATION RIGHTS RESERVED

ADDRESS QUESTIONS TO:

DR. ANDREW R. SASS

IEEE, L.I. SECTION

arshacker@aol.com

This Tech Bulletin consists of two parts. Part 1 describes Code containing a PCINT  with a PW Filter for the IR Sensor. Part 2 describes the Code controlling the signal LEDs on the Mouse for Option 2. This Code  has been streamlined and modified and the signals have been simplified.

Part 1:

This Tech Bulletin is for Mouse designers who have required the information in Tech Bulletin # 5. That Bulletin solved the problem of requiring an IR Sensor interrupt in addition to HW interrupts, Pin 2,3 because they were already using Pins 2,3 for wheel encoders.

Those designers who then read Tech Bulletin #6, learned how to use PW Filters to protect against IR noise spikes such as those originating from Time of Flight IR LEDs.

This brief Tech Bulletin #7 merely combines the PCINT information in Tech Bulletin # 5 with the PW Filter information in Tech Bulletin #6. The resulting Mouse code, Mouse_SIM _GH_3, contains an IR noise immune IR Sensor PCINT, which is IR noise immune, triggered from Pin 10. This leaves HW interrupts on Pin 2,3 free for wheel encoder purposes.

The code in Mouse_SIM _GH_3 is as follows. In the Setup, Group 0 is selected, and Pin 10 is set as the PCINT interrupt pin. The interrupt vector is coded as follows.

Upon sensing the Beacons’ 5V 40 kHz modulated pulse, the IR Sensor emits a negative-going pulse from 5V to 0V and then returns to 5V when the Beacon pulse returns to 0. As a result, Pin 10 first has a negative-going (falling) edge and then a positive-going (rising) edge. The PCINT falling edge initiates the timer start and the rising edge determines the elapsed time. If this elapsed time is 25 ms +- 2ms, the signal is judged to be valid and is processed accordingly. Otherwise, incoming signals continue to be tested and ignored until a valid signal is received.

Part 2:

The Option 2 Code in Mouse_SIM _GH_3 controls the LEDs as follows.

During the Mouse search-

RED LED ON, GREEN LED OFF

Beacon signal sensed-

RED LED ON, GREEN LED ON

Time delay while Mouse moves to trigger proximity sensor.

After proximity sensor triggered-

RED LED OFF, GREEN LED ON

Option 1 Mouse Code is as previously.

During Search-RED LED ON, GREEN LED OFF

Beacon extinguished-RED LED OFF, GREEN LED ON

---------------------------

As in Tech Bulletin #1 the Beacon’s LEDs….

Beacon ON

RED LED ON, GREEN LED OFF

Beacon OFF

RED LED OFF, GREEN LED ON